C.1	Introduction

This Appendix contains checklists and ACEA Performance Data Set forms that may be used to ensure correct documentation of ACEA performance claims.

All lubricant marketers are required to prepare an ACEA Performance Data Set for each formulation making an ACEA performance claim.  An example is given in Form C.3.  This Data Set is derived from the Candidate Data Package and, if appropriate, the Programme Extension Data, as described in 6.1; the Candidate Data Package and any additional programme extension data contain a complete record of each test development programme conducted under the Code and is required to confirm the performance of an engine lubricant against the relevant ACEA Oil Sequence(s).

Form C.1	Candidate Data Package checklist for ACEA Oil Sequence conformance

Form C.2	Programme Extension Data checklist for ACEA Oil Sequence conformance

Form C.3	ACEA Performance Data Set for ACEA Oil Sequence qualification

Part A: Details of lubricant marketer and engine lubricants

Part B: Laboratory tests

Part C: Engine test results

Part D: Qualification conformance

Form C.1 Candidate Data Package checklist for ACEA Oil Sequence conformance

Conducted by: Additive package designation: Viscosity grades: Lubricant codes: ATC data package reference number (if available): Included in this Data Package Yes No 1 Laboratory tests for formulations listed above 2 Formulations for all test lubricants 3 Results of all registered ASTM, JASO and CEC engine tests 4 Test declared ‘out of control’ 5 Test(s) declared ‘not available’ 6 Applicable test stand reference data 7 Properties and identity of base stocks used 8 Formulation modifications and read-across documentation 9 Test programme design document

Signed on behalf of (company): Function: Authorised name: Authorised signature: Date: Company reference document number:

Form C.2 Programme Extension Data checklist for ACEA Oil Sequence conformance

Checklist for engine lubricant development programme

Lubricant Code(s): Viscosity grade(s) covered by this Data Package:

Included in this Data package Yes No - Form C.1 completed For formulations developed in the programme extension 2 Formulations for all test lubricants 3 Results of all laboratory tests on the final candidates 4 Results of all registered ASTM, JASO and CEC engine tests 5 Test has been declared ‘out of control’ 6 Test(s) declared ‘not available’ 7 Applicable test stand reference data 8 Properties and identity of base stocks used 9 Read-across documentation (VGRA, VMI, BOI)

Signed on behalf of (company): Function: Authorised Name: Authorised Signature: Date: Company reference document number:

| COMPANY LOGO OR STAMP |  | COMPANY CONFIDENTIAL INFORMATION |
| --- | --- | --- |

| - Form C.3 ACEA performance data set for ACEA Oil Sequence qualification |  |
| --- | --- |
| Part A Details of the lubricant marketer and engine lubricants |  |
| Details of lubricant marketer Company: Address: Contact Person: Function: Phone No: Fax No: Email address: |  |
| Lubricant details Brand Name: SAE J300 viscosity grade: Lubricant Code Number: ACEA performance(1): |  |
| Details of any rebrands Brand Name(s): |  |
| (1) List each applicable ACEA Oil Sequence category |  |
| Company Document Ref. No. |  |

| COMPANY LOGO OR STAMP |  | COMPANY CONFIDENTIAL INFORMATION |
| --- | --- | --- |

| ###### Form C.3 ACEA Performance Data Set for ACEA Oil Sequence ###### qualification |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Part B Laboratory tests |  |  |  |  |  |
| Laboratory test | Parameter | Test method | Units | Test result | Limits |
| SAE Viscosity | Kinematic viscosity at 100 °C Low-temperature cranking viscosity Low-temperature pumping viscosity | ASTM D445 ASTM D5293 ASTM D4684 | mm2/s mPa.s mPa.s |  |  |
| Shear Stability | Viscosity after 30 cycles measured at 100 °C | CEC L-14-93 Or ASTM D6278 Or ASTM D7109 | mm2/s |  |  |
| Shear Stability | Viscosity after 90 cycles measured at 100 °C | ASTM D7109 | mm2/s |  |  |
| HTHS Viscosity | Viscosity at 150 °C and 106 s-1 shear rate Viscosity at 100 °C and 106 s-1 shear rate | CEC L-36-A-90 | mPa.s mPa.s |  | ........ Report |
| Evaporative loss | Weight loss after 1 h at 250 °C | CEC L-40-93 | mass % |  |  |
| TBN |  | ASTM D2896 ASTM D4739 | mg KOH/g |  |  |
| Sulphur |  | ASTM D5185 Or ASTM D4951 | mass % |  |  |
| Phosphorus |  | ASTM D5185 Or ASTM D4951 | mass % |  |  |
| Sulphated Ash |  | ASTM D874 | mass % |  |  |
| Chlorine |  | ASTM D6443 | mass % |  |  |
| Oil/Elastomer Compatibility | Max. variation of characteristics after immersion for 7 days in fresh oil without pre-ageing RE-6 Tensile strength Elongation at rupture Volume variation RE-7 Tensile strength Elongation at rupture Volume variation RE-8 Tensile strength Elongation at rupture Volume variation RE-9 Tensile strength Elongation at rupture Volume variation | CEC L-112-16 | % % % % % % % % % % % % |  | Report -70 / +20 Report -65 / +15 Report -51 / + 9 Report -65 / +19 |
| Foaming Tendency | Tendency - stability | ASTM D892 Seq. I (24 C) Seq II (94 C) Seq. III (24 C) | mL mL mL |  | 10 - nil ......... 10 - nil |
| High Temp Foaming | Tendency - stability | ASTM D6082 Seq. IV(150 C) | mL |  |  |
| Oil Oxidation with Biodiesel for Engine Oils operating in the presence of Biodiesel Fuel | Oil Oxidation @ 168h (DIN 51453) Oil Oxidation @ 216h (EOT) (DIN 51453) Viscosity Increase, relative at 168h (Delta KV100) Viscosity Increase, relative at 216h (Delta KV100 at EOT 216h) | CEC L-109-14 | A/cm A/cm % % |  |  |
| Oxidation | Oxidation Induction time (PDSC) | CEC-L-085-99 | min |  |  |
| Low temperature pumpability | MRV Yield stress (MRV at SAE J300 temperatures applicable for the fresh oil viscosity grade) | CEC-L-105-12 | mPas Pa |  |  |
| Corrosion | Copper increase Lead increase Copper strip rating | ASTM D6594 | ppm ppm max |  |  |
| Company Document Ref. No. |  |  |  |  |  |

| COMPANY LOGO OR STAMP |  | COMPANY CONFIDENTIAL INFORMATION |
| --- | --- | --- |

| Form C.3 ACEA performance data set for ACEA Oil Sequence qualification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Part C Engine test results – light duty engines |  |  |  |  |  |  |
| Engine test | Parameter | Test method | Units | Test result | Ref. oil result | Limits |
| EP6CDT | Piston Cleanliness Turbo charger deposits | CEC L-111-16 | merit merit |  |  |  |
| Sequence IVB | Average intake lifter volume loss (8 position average) End of test Iron | ASTM D8350 | mm3 ppm |  |  |  |
| Sequence VH | Average engine sludge Rocker cover sludge Average piston skirt varnish Average engine varnish Compression ring (hot stuck) Oil screen clogging | ASTM D8256 | merit merit merit merit % |  |  | ³ 7.6 ³ 7.7 ³ 8.6 ³ 7.6 none Report |
| M271 EVO | Engine sludge, average | CEC L-107-19 | merit |  |  | ³ 8.3 |
| M111 | Fuel economy improvement vs reference oil RL 191 (SAE 15W-40) | CEC L-54-96 | % |  |  |  |
| JASO FE | Fuel economy improvement | JASO FE M366 (Toyota 2ZR-FXE) | % |  |  | ≥ 0.0 |
| TOYOTA 1KD-FTV | Turbo Charger Compressor Deposit | CEC L-114-19 | merit |  |  | ≥ 25 |
| Sequence IX | Low Speed Pre-Ignition events Average number for 4 iterations Number of events per iteration | ASTM D8291 | Number Number |  |  | ≤ 5 ≤ 8 |
| Sequence X | Chain wear GDI Elongation of timing chain | ASTM D8279 | % |  |  | ≤ 0.085 |
| Company Document Ref. No. |  |  |  |  |  |  |

| COMPANY LOGO OR STAMP |  | COMPANY CONFIDENTIAL INFORMATION |
| --- | --- | --- |

| Form C.3 ACEA performance data set for ACEA Oil Sequence qualification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Part C Engine test results - light-duty diesel engines |  |  |  |  |  |  |
| Engine test | Parameter | Test method | Units | Test result | Ref. oil result | Limits |
| DV6C | Absolute viscosity increase at 100 °C and 5.5 % soot Piston merit | CEC L-106-14 | mm2/s merit |  |  | £ 0.9xRL248 ³ 2.5 |
| VW TDI | Piston cleanliness Cylinder-spreading limit No ring sticking, max for any ring | CEC L-117-20 | merit merit ASF |  |  | ≥ RL276 – 5 ≤ 13 0 |
| OM646LA | Cam wear outlet, average Cam wear inlet, average Cylinder wear, average Bore polishing Tappet wear inlet, average Tappet wear outlet, average Piston cleanliness, average Engine sludge, average | CEC L-099-08 | µm |  |  |  |
| Company Document Ref. No. |  |  |  |  |  |  |

| COMPANY LOGO OR STAMP |  | COMPANY CONFIDENTIAL INFORMATION |
| --- | --- | --- |

| ###### Form C.3 ACEA performance data set for ACEA Oil Sequence qualification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Part C Engine test results - heavy-duty diesel engines |  |  |  |  |  |  |
| Engine test | Parameter | Test method | Units | Test result | Ref. oil result | Limits |
| OM646LA | Cam wear outlet, average | CEC L-099-08 | µm |  |  |  |
| Mack T8-E | Relative viscosity at 4.8 % soot | ASTM D5967 |  |  |  | £ 2.1 |
| OM471 | Piston cleanliness (grooves and piston undercrown), average Oil consumption | CEC L-118-21 | % g/h |  |  |  |
| CAT 1N | Weighted demerits (WDN) Top Groove Fill (TGF) Top Land Heavy Carbon (TLHC) Oil Consumption (0-252h) Piston, ring and liner scuffing Piston ring sticking | ASTM D6750 | Demerit % % g/kWh |  |  |  |
| CAT C13 | Merit rating Hot stuck rings | ASTM D7549 | Merit |  |  |  |
| Cummins ISM | Merit rating Top ring mass loss Crosshead, weight loss Oil Filter Diff. Press at 150 h Engine sludge Adj. screw weight loss | ASTM D7468 | Merit mg mg kPa merit mg |  |  |  |
| Mack T-12 | Merit Average Liner wear Average top ring weight loss End of test lead concentration, mass fraction Delta lead 250-300 h, mass fraction Oil consumption phase 1 | ASTM D7422 | µm mg ppm ppm g/h |  |  |  |
| OM646LA BIO | Piston cleanliness Ring sticking Sludge | CEC L-104-16 | merit ASF merit |  |  |  |
| Volvo T-13 | KV increase (300-360h) Oxidation peak hight Nitration peak hight Oil consumption (avg 48-192h) | ASTM D8048 | % A/cm A/cm g/h |  |  |  |
| COAT | Aeration | ASTM D8047 | % |  |  |  |
| Company Document Ref. No. |  |  |  |  |  |  |

| COMPANY LOGO OR STAMP |  | COMPANY CONFIDENTIAL INFORMATION |
| --- | --- | --- |

###### Form C.3 ACEA performance data set for ACEA Oil Sequence qualification

Part D Qualification conformance

I hereby attest to using and satisfying the guidelines as defined in ATIEL Code of Practice. Name of Authorised Company Representative: Function: Phone no: Email address: Signature of Authorised Company Representative: Date: Company document reference no: